# Engine Live Data Programmer Implementation Guide

Implement one interactive engine demonstration section inside the existing virtual automotive platform. Students change virtual sensor signals and operating switches, then observe the calculated commands, state sequence, animations and live trends.

Use Codex_EngineLab_Prompt.md as the authoritative build instruction. It includes the complete TypeScript reference, stable input and output types, interface requirements and acceptance checks. This guide explains the expected result; MATLAB is not needed to run the converted section.

## Integration and study boundary

Inspect the existing repository first. Reuse its routing, navigation, components, styles and state conventions. Build no new platform and redesign no unrelated section. The physical 1NZ setup is separate and has no electric water pump. Virtual and physical stages are tested independently, with no comparison.

## Approved input data

| Input | Unit | Range | Default |
| --- | --- | --- | --- |
| Battery voltage | V | 8 to 14.9 | 13.8 |
| Engine speed imposed | rpm | 0 to 6500 | 800 |
| Throttle position | % | 0 to 100 | 2 |
| Manifold absolute pressure | kPa | 20 to 105 | 35 |
| O2 sensor voltage | V | 0.05 to 0.95 | 0.45 |
| Virtual ECT1 voltage | V | 0.2 to 4.8 | 1.0 |
| Virtual ECT2 voltage | V | 0.2 to 4.8 | 1.1 |
| Accessory load | % | 0 to 100 | 10 |

Six binary inputs begin at 0: ignition, simulated authorisation, start request, stop request, emergency stop and A/C request. Use switches and clearly labelled actions. Start and stop are momentary requests; emergency remains latched until deliberately cleared.

Cap finite inputs before calculation and display the accepted value. Reject blank or invalid entries while keeping the last accepted value. A change to one input must preserve all other inputs and the current model state.

# Signal conversion and fuel calculation

Evaluate each 0.05-second sample in this order: cap inputs, update Start/Run once, convert both ECT voltages, then calculate fuel, idle, ignition and cooling using the updated state. Publish one shared snapshot to the whole interface.

## Virtual coolant temperature conversion

Use the exact piecewise conversion in the prompt. Lower voltage represents higher temperature. The following checkpoints explain its shape; a single straight-line conversion is not acceptable.

| Voltage in V | Temperature in degrees C |
| --- | --- |
| 4.8 | -20 |
| 4.3 | 0 |
| 3.5 | 25 |
| 2.5 | 50 |
| 1.5 | 75 |
| 1.0 | 90 |
| 0.6 | 105 |
| 0.3 | 125 |
| 0.2 | 135 |

## Fuel and injection

The expressions below use clip(value, minimum, maximum). RPM, TPS, MAP, O2 and battery are accepted inputs. ECT1 is the converted temperature. Percent corrections are divided by 100 only where shown.

load = clip(0.55*((MAP-20)/85)*100 + 0.45*TPS, 0, 100)
lambda = clip(1 + (0.45-O2)*0.35, 0.75, 1.25)
closed = injectionEnable && ECT1>=70 && RPM>=600
STFT = closed ? clip((lambda-1)*120, -25, 25) : 0
warmup = clip((70-ECT1)*0.5, 0, 40)
batteryComp = clip((13.8-battery)*0.18, 0, 1.2)
basePW = clip(1+0.00035*RPM+0.025*(MAP-20)+0.012*TPS, 1, 15)
PW = injectionEnable
   ? clip((basePW+batteryComp)*(1+(STFT+warmup)/100), 0, 20)
   : 0

Show pulse widths and battery compensation in ms; STFT, warm-up enrichment and load in percent; lambda as a ratio; and closed loop as an active/inactive status. Lambda is a simplified model estimate, not a calibrated physical measurement.

# Idle ignition and cooling calculation

## Idle and ignition

target = 750 + clip((80-ECT1)*5.625, 0, 450)
idleActive = state==RUN && TPS<=5 && RPM<2000
error = idleActive ? target-RPM : 0
idleDuty = idleActive ? clip(25+0.06*error+0.25*accessory,0,100) : 0
timing = !ignitionEnable ? 0 : state==CRANK ? 5
       : clip(10+0.004*(RPM-800)-0.08*(MAP-35)+0.015*error,-5,45)

Show actual and target RPM together, signed RPM error, idle duty in percent, timing in degrees BTDC and idle status. Negative timing represents ATDC. No idle duty or other command automatically changes the imposed RPM.

## Cooling commands

Let T be max(ECT1, ECT2) and difference be abs(ECT1-ECT2), in degrees C. First, emergency stop or accepted battery below 8 V forces both duties to zero. Otherwise, difference greater than 15 degrees C forces both to 100%. Otherwise apply these schedules.

| Command | Temperature condition | Duty percent |
| --- | --- | --- |
| Fan | T < 88 | 0 |
| Fan | 88 <= T < 95 | 20 + (T-88)*(30/7) |
| Fan | 95 <= T < 105 | 50 + (T-95)*5 |
| Fan | T >= 105 | 100 |
| Pump in RUN | T < 50 | 35 |
| Pump in RUN | 50 <= T < 90 | 35 + (T-50)*(35/40) |
| Pump in RUN | 90 <= T < 110 | 70 + (T-90)*1.5 |
| Pump in RUN | T >= 110 | 100 |
| Pump outside RUN | T > 95 | 60 |
| Pump outside RUN | T <= 95 | 0 |

With A/C requested and state code at least 2, fan duty has a 35% minimum. This includes SHUTDOWN code 4. Cap final duties to 0–100%. Cooling active means either duty is above zero.

Keep the model details: exactly 15 degrees C does not trigger disagreement; cooling may run while the engine is OFF; emergency overrides disagreement. The below-8-V check is unreachable after the 8-V input cap, but remains in the translated logic.

# Start sequence and software contract

State codes are OFF 0, PRIME 1, CRANK 2, RUN 3 and SHUTDOWN 4. Add 0.05 to state elapsed before evaluating one transition; reset elapsed on a transition. Preserve the original double-precision comparisons and branch order.

| State | Transition conditions in priority order |
| --- | --- |
| OFF | Enter PRIME when ignition, authorisation and start are true, emergency is false and battery is at least 9.5 V. |
| PRIME | Emergency, stop or ignition off: SHUTDOWN. Otherwise elapsed at least 2 s: CRANK. |
| CRANK | Emergency, stop, ignition off or battery below 8.5 V: SHUTDOWN. Otherwise RPM at least 500: RUN. Otherwise elapsed at least 5 s: SHUTDOWN. |
| RUN | Emergency, stop or ignition off: SHUTDOWN. There is no automatic low-RPM stall or authorisation-loss shutdown. |
| SHUTDOWN | Elapsed at least 0.5 s: OFF. A held start request may subsequently retrigger. |

Fuel pump is enabled in PRIME, CRANK and RUN; starter only in CRANK; injection and ignition in CRANK and RUN. StartPermitted means sequence active. SafeStartCondition includes the momentary start request, so it can be false during a normal RUN.

## Stable JSON compatible data

The TypeScript core defines 14 named inputs, a persistent state object and a ModelSnapshot. Each snapshot contains accepted inputs, capped-field flags, internal state, simulation time, and 30 numeric outputs plus the readable state label. Preserve the field names or map them through one explicit host adapter.

let state = initialState();
const sample = stepEngineModel(fullInputs, state);
state = sample.internalState;
// Render values, graphs and animation from sample.

Initial state has sampleIndex -1. The first call produces sample 0 at time 0, matching Simulink. Running through time 3 s inclusive requires 61 calls. State elapsed accumulates in floating point; do not round it or introduce epsilon thresholds.

## Simulation controls

Run, Pause, Resume and Step control simulation time. Start engine and Stop engine create 10-sample request pulses and do not set states directly. Stop clears a pending start. Emergency stays latched. Reset all pauses, restores defaults and clears history. Presets start a fresh experiment but preserve a latched emergency until deliberately cleared.

Paused readouts remain the last sampled values. Mark edited inputs as pending until Step or Resume. Use one timer owner; pause when hidden and avoid catch-up jumps. Input edits and UI re-renders must not reset the operating state.

# Required interface and graphical result

Deliver a usable section with grouped input controls, a state sequence strip, numerical cards, four detail panels, live trends, event history and CSV export. Reuse the host platform design and adapt the layout for narrow screens.

Layout example at warm RUN. It describes the information arrangement; the completed platform must provide real interactive controls and calculated traces.

## Animation driven by model outputs

Use an original lightweight engine cutaway or SVG schematic. A stationary car silhouette may locate the engine compartment. Fuel pump, starter, injection and ignition indicators follow their enable signals. Show pulse width and ignition timing numerically alongside the illustration.

Fan and virtual electric-pump motion follows their calculated duties, including cooling while OFF. RPM motion is illustrative and occurs only in CRANK or RUN with imposed RPM above zero. Label all motion illustrative; duty is a command, not measured motor speed. Do not invent road movement, acceleration or thermal feedback.

Show both virtual ECT temperatures, their difference and the active cooling response. Add a simple legend separating control signals from illustrative flow. Do not assign unmodelled physical sensor positions. Pause freezes animation; reduced-motion mode retains all labels and bars.

## Live trends and output access

Fuel panels expose pulse width, STFT, lambda, closed loop, load, base pulse, enrichment and battery compensation. Idle panels expose actual/target RPM, error, duty, timing and active status. Cooling panels expose both temperatures, fan/pump duty, difference, control temperature and statuses. Start/Run exposes state, elapsed time, relays, enables and start flags.

Every graph has a visible trace legend, units and simulation seconds. Use separate panels for unlike units. Keep a rolling 60-second sampled history, readable while paused, and export that retained window to CSV. Provide keyboard access, clear focus and an accessible data alternative.

# Manual checks and completion requirements

Select a preset, run the simulation, enable ignition and simulated authorisation, and press Start engine. Verify values after reaching RUN unless a check states otherwise. Change one parameter at a time and confirm the graphic, number and graph agree.

| Scenario | Expected key results |
| --- | --- |
| Warm default inputs | Injector 1.679 ms; lambda 1; STFT 0%; target 750 rpm; error -50 rpm; idle duty 24.5%; timing 9.25 degrees BTDC; fan 28.5714%; pump 70%. |
| O2 voltage 0.10 V | Lambda 1.1225; STFT +14.7%; injector 1.925813 ms. A live edit preserves RUN. |
| O2 voltage 0.90 V | Lambda 0.8425; STFT -18.9%; injector 1.361669 ms. |
| ECT1 1.0 V and ECT2 2.0 V | 90 and 62.5 degrees C; difference 27.5; disagreement active; fan and pump 100% unless emergency is active. |
| Both ECTs 3.5 V in RUN | About 25 degrees C; closed loop off; enrichment 22.5%; injector 2.056775 ms; target 1059.375 rpm; pump 35%. |
| Battery 9 V before start | Start remains blocked in OFF. Warm-signal cooling can still run. |
| RPM 200 during start | CRANK times out into SHUTDOWN. With held start, sample 7.4 s is SHUTDOWN; the ordinary pulse avoids later retriggering. |
| Emergency during RUN | Next processed sample enters SHUTDOWN. Relays/enables and both cooling duties become zero. |
| Both ECTs 0.6 V while OFF | Temperature 105 degrees C, fan 100%, virtual pump 60%, unless emergency is active. |

Also check every input cap, invalid-entry rejection, exactly 15-degree disagreement, A/C during SHUTDOWN, pause/resume, reset, presets, legends, responsive layout and CSV sample alignment. Full preset inputs and precise review values are in the master prompt.

## What the programmer must deliver

Report changed files, run instructions and the integrated route. Show a screenshot or live preview of the completed interface, engine graphic and labelled live trends. Report checks actually performed and any limitations. Neither a logic-only implementation nor a static picture completes this task.

The handover reference is checked against the 12 saved engine-model cases. The platform programmer must repeat the relevant numerical and interaction checks after integration. These are software checks, not a comparison with the physical 1NZ stage.

Open the shared Engine Diagnostic Demonstration conversation

The link is an optional snapshot. The master prompt is self-contained and includes both conversation IDs, so unavailable chat access does not block implementation.